Smart 5g Optimisation and deployment project

Date:January-April 2024

Network Parameter Tuning Framework Design

Title: Adaptive Parameter Tuning Framework for 5G RAN and Core Network

A. Adjustable Parameters

| Parameter | Description | Range | Default Value | Tuning Frequency |
| --- | --- | --- | --- | --- |
| Transmission Power (dBm) | Power level of base station radios | 10 – 50 dBm | 43 dBm | Real-time / hourly |
| Handover Margin (dB) | Threshold for triggering handovers | 0 – 10 dB | 3 dB | Daily / event-driven |
| Beamforming Weights | Directional antenna pattern coefficients | Continuous vector parameters | Initialized from training | Weekly / monthly |
| Scheduler Weights | Priority weights for traffic classes (e.g., URLLC) | 0 – 1 per class | 0.5 for default | Per deployment |
| Slice Bandwidth Allocation | Amount of bandwidth assigned per network slice | 5 – 100 MHz | According to SLA | Real-time / SLA changes |
| Load Balancing Thresholds | Trigger points for traffic offloading | 60% – 90% utilization | 75% utilization | Real-time monitoring |

B. Thresholds and Policies

- Handover Policy:

  - Trigger handover if RSRP difference > 3 dB for more than 200 ms

  - Prioritize target cell with lowest load < 70% utilization

- Power Control Policy:

  - Reduce power by 1 dBm in low-load periods >30 mins to save energy

  - Increase power if packet loss > 2% persistently for 10 mins

- Scheduling Policy:

  - URLLC traffic prioritized with max delay ≤1 ms

  - eMBB traffic throughput maximized during off-peak hours

- Slice Management Policy:

  - Reallocate bandwidth dynamically based on slice SLA breach alerts

  - Minimum guaranteed bandwidth per slice maintained at 80% SLA

C. Framework Architecture

- Monitoring Module: Continuously collects KPIs and feeds into analytics engine.

- Decision Engine: ML models predict optimal parameter settings based on current network state.

- Parameter Adjustment API: Interface to RAN and Core controllers for dynamic updates.

- Feedback Loop: Performance metrics post-adjustment fed back for continuous learning.

Manual Override: Network ops can override auto-t